Зміст

Agents and Attributes	2

Environment Attributes:	2

Agents	6

Formulas	8

Stroke 1	8

Stroke 2 & Stroke 3	9

Stroke 4	13

BEHAVIOUR	17

initial data	23

Single Cylinder Model

Agents and Attributes

Environment – engine cylinder

Agents – air, fuel, working mixture, combustion products. Agent type – SUBSTANCE.

Environment Attributes:

| № | Accepted Notation | Parameter Description | Measurement Units | Attribute Name in the Model | Initial Value1 |
| --- | --- | --- | --- | --- | --- |
|  | D | Cylinder diameter | m | cylinderDiameter | 0.46 |
|  | S | Piston stroke | m | pistonStroke | 2*crankRadius |
|  | L | Connecting rod length | m | connectingRodLength | 1.374 |
|  | R | Crank radius | m | crankRadius | 0.29 |
|  | l | the length by which the axis of the crank is shifted relative to the axis of the cylinder | m | crankOffsetLength | 0 |
|  |  | Compression ratio |  | compressionRatio | 14 |
|  | Vc | Combustion chamber volume | m3 | combustionChamberVolume | ((cylinderDiameter)2* PI*crankRadius/2)/ (compressionRatio-1) |
|  | Vp | The volume of the supra-piston cavity | m3 | cylinderSupraPistonVolume | - |
|  | dVp | Change in the volume of the supra-piston cavity at the calculated time interval, caused by the movement of the piston | m3 | cylinderSupraPistonVolumeD | - |
|  | n | Crankshaft rotation frequency | min-1 | crankshaftRotationFrequency | 495 |
|  | Cm | Average piston speed | m/s | averagePistonSpeed | 4*crankRadius* crankshaftRotationFrequency |
|  | ∆z | Piston clearance at TDC position | m | pistonClearance | 0.0008 |
|  | Dinl | Inlet valve diameter | m | inletValveDiameter | 0.165 |
|  | Dexh | Exhaust valve diameter | m | exhaustValveDiameter | 0.16 |
|  | Vint | Intake manifold wolume | m3 | intakeManifoldVolume | 3.210 |
|  | Vexh | Exhaust manifold wolume | m3 | exhaustManifoldVolume | 2.350 |
|  | μexhfexh | The effective cross-section of the exhaust valves | m2 | exhaustValveCrossSectionalArea | - |
|  | μinlfinl | The effective cross-section of the inlet valves | m2 | inletValveCrossSectionalArea | - |
|  | (μinlfinl)max | The maximum value of the inlet valve cross-section | m2 | maxInletValveCrossSectionalArea | (inletValveDiameter)2 * PI/4 |
|  | (μexhfexh)max | The maximum value of the exhaust valve cross-section | m2 | maxExhaustValveCrossSectionalArea | (exhaustValveDiameter)2 * PI/4 |
|  | φ | The value of the crank rotation angle |  | crankshaftRotationAngle | 0 |
|  | φ1 | The angle of rotation of the crankshaft corresponding to the opening of the exhaust valve | CRA | exhaustValveOpenAngle | 475 |
|  | φ2 | The angle of rotation of the crankshaft corresponding to the opening of the inlet valve | CRA | inletValveOpenAngle | 690 |
|  | φ3 | The angle of rotation of the crankshaft corresponding to the closing of the exhaust valve | CRA | exhaustValveCloseAngle | 20 |
|  | φ4 | The angle of rotation of the crankshaft corresponding to the closing of the inlet valve | CRA | inletValveCloseAngle | 220 |
|  | φinlOpenD | Inlet valve opening duration | CRA | inletValveOpenDuration | 245 |
|  | φexhOpenD | Exhaust valve opening duration | CRA | exhaustValveOpenDuration | 210 |
|  | φinlCloseD | Inlet valve close duration | CRA | inletValveCloseDuration | 20 |
|  | φexhCloseD | Exhaust valve close duration | CRA | exhaustValveCloseDuration | 20 |
|  | φinj | The angle of rotation of the crankshaft corresponding to the start of fuel injection | CRA | fuelInjectionStartAngle | 345 |
|  | φcomb | The angle of rotation of the crankshaft corresponding to the start of combustion | CRA | startCombustionAngle | 355 |
|  | ∆φz | Crankshaft rotation angle corresponding to combustion duration | CRA | combustionDuration | 50 |
|  | φz | The crankshaft rotation angle corresponding to the end of the combustion process | CRA | stopCombustionAngle | startCombustionAngle + combustionDuration |
|  | Tp | Average piston surface temperature | К | pistonTemperature | 673 |
|  | Th | Average cylinder head surface temperature | К | cylinderHeadTemperature | 723 |
|  | Tc | Average cylinder surface temperature | К | cylinderWallSurfaceTemperature | 723 |
|  | Fp | Piston bottom surface area | m2 | pistonArea | (cylinderdiameter)2*PI/4 |
|  | Fh | Cylinder head surface area | m2 | cylinderHeadArea | pistonArea + PI*cylinderDiameter * pistonClearance |
|  | Fc | Cylinder surface/wall area above the piston | m2 | cylinderWallSurfaceArea | - |
|  | pi | Pressure in cylinder |  | pressureInCylinder | 180*103 |
|  | Ti | Temperature in cylinder |  | temperatureInCylinder | 873.15 |
|  | pinl | Gas pressure in the inlet channel |  | pressureInInletValve | 200*103 |
|  | pexh | Gas pressure in the exhaust channel |  | pressureInExhaustValve | 103*103 |
|  |  |  |  | stoichiometricAirFuelRatio | 14.5 |
|  | H | mass fraction of H element in fuel |  | H | 0.125 |
|  | O | mass fraction of O element in fuel |  | O | 0.005 |
|  | C | mass fraction of C element in fuel |  | C | 0.87 |
|  | rО2 | The content of oxygen in the air | kg | oxygenInAirCoef | 0.21 |
|  | M0' | The amount of air that is theoretically required to burn 1 kg of fuel | kmol /kg | airNeededAmountForCombustion | (1/oxygenInAirCoef)*((C/12) + (H/4) + (O/32)) |
|  | CT | The tangential component of the velocity of the working fluid relative to the surface of the cylinder walls | m/s | WMvelocityTangentialComponent | 10 |
|  | Rμ | Universal gas constant | kJ/(kmol K) | gasConstant | 8.314 |
|  | aair, acomb, bair, bcomb | Constant coefficients in the interpolation dependences of molar heat capacity on temperature for a given composition of the working fluid in the given temperature range (for air and combustion products) |  | If T - 273 <= 1000: coefA1 == 3.653 coefA2 == 1.957 coefB1 == -1.337*10-3 coefB2 == 0.368*102 ki=1+8,314aair11-rγi+acomb1rγi+bair11-rγi+bcomb1rγi(Ti-273) If T - 273 >1000: coefA12 == 2.415 coefA22 == 1.287 coefB12 == 1.257*10-3 coefB22 == 0.456*102 ki=1+8,314aair21-rγi+acomb2rγi+bair21-rγi+bcomb2rγi(Ti-273) |  |

Agents

To be able to represent different types of fuel (which will allow us to consider different engine models in the future), air and combustion products, which are an integral part of the gas exchange process in the engine cylinder, we introduce the agent type SUBSTANCE.

The main attributes of substance agents are: the current mass and temperature of the substance in the engine elements, the volume in the cylinder, the amount of substance in moles, density, the speed of movement of the substance in the engine elements, the gas constant, the heat of the gas, the heat capacity of the substance (at constant pressure and constant volume), the adiabatic index, etc. The chemical structure of a substance is given by a set of MOLECULE type agents, which, in turn, contain a functional that reflects the set of atoms that make up the given molecules. Such a representation will allow us to further consider the processes of mixture formation and combustion at the quantum-chemical level, and, accordingly, to model the possible effects of the qualitative and quantitative composition of fuel and fuel mixtures on engine operation.

SUBSTANCE:obj(

mass:real,

massInCylinder:real,

massInInletValve:real,

massInExhaustValve:real,

volumeInCylinder:real,

substanceAmount:real,

density:real,

velocityInInletValve:real,

velocityInExhaustValve:real,

gasСonstant:real,

gasHeat:real,

temperatureInInletValve:real,

temperatureInExhaustValve:real,

temperatureInCylinder:real,

heatCapacityAtConstantPressure:real,

heatCapacityAtConstantVolume:real,

heatCapacityRatio: real,

moleculesID:(int)->int,

moleculesNum:int

)

| № | Accepted Notation | Parameter Description | Measurement Units | Attribute Name in the Model | Initial Value |
| --- | --- | --- | --- | --- | --- |
| air |  |  |  |  |  |
|  | Tinl | Air temperature in the inlet valve |  | temperatureInInletValve | 315 |
|  | Rf | Gas constant for fresh charge (air) | J /(kgК) | gasConstant | 287.2 |
|  | Gair | Air mass in cylinder |  | massInCylinder | 0 |
|  | kinl | The average value of the adiabatic index for the working fluid at the inlet segment |  | heatCapacityRatio | 1.4 |
|  | ρair | air density | kg/m3 | density | pressureInInletValve/(air.gasConstant*air.temperatureInInletValve) |
| combustionProducts |  |  |  |  |  |
|  | Rc | Gas constant for combustion products | J /(kgК) | gasConstant | 290 |
|  | kexh | The average value of the adiabatic index for the combustion products at the outlet segment |  | heatCapacityRatio | 1.25 |
|  | Tcomb | combustion products temperature in cylinder |  | temperatureInCylinder | 873.15 |
|  | Gcomb | combustion products mass in cylinder |  | massInCylinder | (pressureInCylinder*(((cylinderDiameter)2*piConst* crankRadius/2)/ (compressionRatio-1))) /(combustionProducts.gasConstant*temperatureInCylinder |
| workingMixture |  |  |  |  |  |
|  | Texh | gas mixture temperature in the exhaust valve | К | temperatureInExhaustValve | 773.15 |
|  | RexhInl | Gas constant for a gas mixture | J /(kgК) | gasConstant | 290 |
|  | ρwm | gas mixture density in cylinder | kg/m3 | density | pressureInCylinder/(workingMixture.gasConstant*temperatureInCylinder) |
| fuel |  |  |  |  |  |
|  | ρf | Fuel density before the nozzle channel | kg/m3 | density | 830 |
|  | Bcycle | Cyclic supply of fuel | kg/ cycle | fuelCyclicSupply | 25 |
|  | Qlow | Lowest heat of fuel combustion | kJ /kg | fuelCombustionLowestHeat | 42.7*106 |

Formulas

- The value of the crank rotation angle

dφdτ=πn30

- The effective cross-section of the valves

μf(φ)=0,5 μfmax 1-cosφ-φopenφopenD±∆φcorr∙π

μf(φ)=0,5 μfmax 1+cosφ-φcloseφcloseD±∆φcorr∙π

- The volume of the supra-piston cavity

Vpφ=Vc+πD24R+L2-e2-L2-e+Rsin(φ-β)2-Rcos(φ-β)

- The amount of heat transferred from the cylinder walls to the working fluid or from the working fluid to the walls of the supra-piston cavity:

dQh=αhFpT-Tp+FhT-Th+Fc(T-Tc)dτ

Stroke 1

- The mass of air entering the suprapiston cavity from the inlet valve:

dGinlAir=pinlRairTinlppinl1kairμinlfinl2kairkair-1RairTinl1-ppinlkair-1kairdτ

- The mass of gases (here a mixture of air and combustion products) leaving the cylinder into the exhaust valve:

dGexhMix=pRmixT(pexhp)1kmixμexhfexh2kmixkmix-1RmixT1-(pexhp)kmix-1kmixdτ

- The mass of air leaving the cylinder into the inlet valve:

dGairInl=pRairTpinlp1kairμinlfinl2kairkair-1RairT1-pinlpkair-1kairdτ

- Change in the pressure of the gas mixture in the cylinder cavity above the piston:

  - If ((φinlOpen ≤φ<φinlClose) && (φexhOpen ≤ φ<φexhClose)) &&( pexh< p && p≤pinl) && Gγ>0:

dp=kmixpVRmixTpdGinlAirTinlT-dGexhMix-kmix-1kmixdQhp-dVp

  - If ((φinlOpen ≤φ<φinlClose) && (φexhOpen ≤ φ<φexhClose)) && pexh< p && p≤pinl && Gγ==0

dp=kairpVRairTpdGinlAir-dGairInl-kair-1kairdQhp-dVp

  - If ((φinlOpen ≤φ<φinlClose) && (φ≥φexhClose)) && p≤pinl && Gγ==0

dp=kairpVRairTpdGinlAirTinlT-kair-1kairdQhp-dVp

  - If ((φinlOpen ≤φ<φinlClose) && (φ≥φexhClose)) && p>pinl && Gγ==0

dp=kairpVRairTp(-dGinlAir)-kair-1kairdQhp-dVp

- Temperature:

dT=1GcpdGmixis-i±dQh+Vdp

Stroke 2 & Stroke 3

Algorithm for calculating compression, combustion and expansion processes:

- Set the values of the initial data:

Cylinder diameter – D; Piston stroke – S; Connecting rod length – L; Compression ratio - ε; Crankshaft rotation frequency – n; The coefficient of excess air – α; Piston clearance at TDC position - z; Piston bottom surface area - Fp; Cylinder head surface area - Fh; Average piston surface temperature - Tp; Average cylinder head surface temperature - Th; Average cylinder surface temperature - Tc; The angle of rotation of the crankshaft corresponding to the opening of the exhaust valve - φ1; The angle of rotation of the crankshaft corresponding to the closing of the inlet valve - φ4; The angle of rotation of the crankshaft corresponding to the start of combustion - φcomb; crankshaft rotation angle corresponding to combustion duration - dφz; The amount of air that is theoretically required to burn 1 kg of fuel - M0'; Calculation step - dφi; pressure, volume and temperature of the working fluid in the supra-piston cavity at the moment the inlet valve closes - pv, Vv, Tv; Amount of working fluid at the moment the inlet valve closes - Mv'; Coefficient of residual gases – γ; Coefficients - aair, acomb, bair, bcomb; Elementary fuel composition – C, H, O; Lowest heat of fuel combustion - Qlow; Indicator chart completeness factor - ; Universal gas constant - Rμ; Combustion dynamics indicator - m.

- Calculation of the values of parameters that do not change throughout the entire calculation cycle:

Vh=3,144D2S,    Vc=Vh/(ε-1),

Fp=π4D2,     Fh=π4D2+πD∆z,

Cm=Sn/30,

dτi=dφi/(6n),

McInlv'=Mv'/(1+γ),      Mγv'=McInlv'γ,    rγV=Mγv'/Mv',

M0'=1rO2(C12+H4+O32),

Bcycle=McInlv'/(αM0'),

β0=1+H4+O2αM0'

β=β0+γ1+γ

φz=φcomb+dφz –

- Calculation of the changes in the volume of the supra-piston cavity during the calculated time interval and the surface area of the cylinder walls

φi=φ1=φ4,

Vi=V1=VV,   φi+1=φi+dφi

Vp(i+1)=Vc+0,5Vh1-cosφi+dφi+S8L1-cos2(φi+dφi),

dVpi=Vp(i+1)-Vpi

Fci=πD0,5S1-cosφi+S8L1-cos2(φi+dφi)

- Checking whether the rotation angle value is less than the angle corresponding to the start of the combustion:

  - If the crankshaft rotation angle is less than the angle corresponding to the start of combustion - φi+1≤φcomb

    - calculation of the amount of heat transferred at the calculated segment from the walls of the supra-piston cavity to the working fluid or from the working fluid to the walls

φi=φ1=φ4,  Ti=T1=TV,  pi=p1=pV

αтi=128D0,2(10-5pi)0,8Ti-0,53(2,28+0,308CТCm)Cm0,8

dQhi=αтiFpTi-Tp+FhTi-Th+FciTi-Tcidτ,

    - calculation of the adiabatic index for the working fluid at the beginning of the calculation segment

rγi=rγV

ki=1+8,314aair1-rγi+acombrγi+bair1-rγi+bcombrγi(Ti-273)

    - Calculating the change in pressure of the working fluid dрi, parameters of the working fluid at the end of the calculation section, amount of heat Qh(i+1) and compression work Lс1(i+1) at the end of the calculation segment:

dpi=kipiViki-1kidQТipi-dVpi;   φi+1=φi+dφi;

Vp(i+1)=Vpi+dVpi; pi+1=pi+dpi; Mi+1'=Mv';

Ti+1=pi+1Vi+1/(RμMv'); Qhi+1=Qhi+dQhi;

dLc1(i)=0,5pi+1+pidVi; Lc1(i+1)=Lc1i+dLc1(i)

  - Performing the calculations from p.3 and check the condition φi+1≤φcomb

If the value of the rotation angle is still less than the angle corresponding to the start of combustion then the calculations from p.4.1. will be repeated

The composition of the working fluid in this calculation segment is constant and is characterized by the coefficient of residual gases -  and by the volume fraction of combustion products in the gas mixture - r.

- If the value of the rotation angle is greater than the angle corresponding to the start of combustion - φi+1>φcomb, then must be checked the conditions φi+1>φ1 and φi+1≤φ1

  - If the condition  (φi+1>φcomb) & (φi+1≤φ1) is true, then the following calculations must be performed:

    - φi+1=φcomb=φ1,  Vi+1=Vφcomb=V1

pi+1=pφcomb=p1,  Ti+1=Tφcomb=T1,

ki+1=kcomb=k1, Qh(i+1)=Qhφcomb=Qhcomb,

nc=ln(pcombpv)ln(VvVcomb), Lc1(i+1)=Lc1comb=Lc1

    - Qh(i)=Qhφcomb=Qh1=0; φi+1=φi+dφ

αTi=128D0,2(10-5pi)0,8Ti-0,532,28+0,308CT/CmCm+3,24∙10-3TipiViVhpi-pvVvVikc0,8,

dQhi=αтiFpTi-Tp+FhTi-Th+FciTi-Tcidτ,

    - xi=xφcomb=x1=0, Qx=Qxφcomb=Qx1=0

xi+1=exp-6,908φi-φcombdφzm+1,

dxi=xi+1-xi,   dQxi=BcycleQlowdxi

    - Mγ'=Mγφcomb'=Mγ1'=Mγv'

rγi=rγφcomb=rγ1=Mγv'/Mv',

Mγi'=Mγv'+Mv'βxi,

rγi=Mγi'/Mv'1+(β-1)xi

    - Check the conditions Ti-273≤1000°C or Ti-273>1000°C

    - If the condition Ti-273≤1000°C is true:

ki=1+8,314aair11-rγi+acomb1rγi+bair11-rγi+bcomb1rγi(Ti-273),

    - If the condition Ti-273>1000°C is true:

ki=1+8,314aair21-rγi+acomb2rγi+bair21-rγi+bcomb2rγi(Ti-273),

    - dpi=kipiViVi(β-1)dxi+ki-1kidQxi-dQhipi-dVpi

φi+1=φi+dφi,   pi+1=pi+dpi

Vp(i+1)=Vpi+dVpi,   Mi+1'=Mv'1+(β-1)xi,

Ti+1=pi+1Vp(i+1)/(RμMi+1');

Qhi+1=Qhi+dQhi;  ,   Qxi+1=Qxi+dQxi

dLp1(i)=0,5pi+1+pidVpi;

dLp1i+1=Lp1(i)+dLp1(i)

    - Perform the calculations from p.3 and check the condition (φi+1>φcomb) & (φi+1≤φ1). If the condition is true, then the algorithm of the section 5.1. must be repeated.

  - If the condition (φi+1>φcomb) & (φi+1>φ1) is true, then the following calculations must be performed:

nc=ln(pcombpv)ln(VvVcomb),

pa=pv(Vh+Vc)/Vvnc,

Lc2=(pvVv-pBDCVBDC)/(nc-1),

Lc=Lc1+Lc2,

Qhi+1=Qhφcomb=Qhc,

    - pi+1=pe,   Vi+1=Ve, Vb=Vc+Vh,

pφz=pz,   Vφz=Vz,

np=ln(pzpe)ln(VeVz),    pφb=pb=pe(Vh+Vc)/Venp,

Lp2=(peVe-pbVb)/(np-1),

Lр=Lр1+Lр2

    - Li= Ѱ(Lр-Lc)

pinli=LiVh,    ŋi=LiBcycleQlow

Stroke 4

Algorithm for calculating pressure changes in the supra-piston cavity for the exhaust process:

- Set the values of the initial data:

Cylinder diameter – D; Piston stroke – S; Connecting rod length – L; Compression ratio - ε; Crankshaft rotation frequency – n; Piston bottom surface area - Fp; Cylinder head surface area - Fh; Average piston surface temperature - Tp; Average cylinder head surface temperature - Th; Average cylinder surface temperature - Tc; The angle of rotation of the crankshaft corresponding to the opening of the exhaust valve - φ1; The angle of rotation of the crankshaft corresponding to the opening of the inlet valve - φ2; The angle of rotation of the crankshaft corresponding to the closing of the exhaust valve - φ3; The angle of rotation of the crankshaft corresponding to the closing of the inlet valve - φ4; Calculation step - ∆φi; The maximum value of the exhaust valve cross-section - (μexhfexh)max; The maximum value of the inlet valve cross-section - (μinlfinl)max; Exhaust valve lift duration - φexhLiftD; Exhaust valve duration - φexhD; Exhaust valve close duration - φexhClosD; Correction for the duration of the exhaust valve lift and closing in the approximating dependencies - ∆φexhCorr;

Inlet valve lift duration - φinlLiftD; Inlet valve duration - φinlD; Inlet valve close duration - φinlClosD; Correction for the duration of the inlet valve lift and closing in the approximating dependencies - ∆φinlCorr; Gas pressure and temperature in the exhaust channel - pexh and Texh; The average value of the adiabatic index for the working fluid at the outlet segment - kexh; Gas constant for combustion products - Rc; Gas pressure and temperature in the inlet channel - pinl and Tinl; The average value of the adiabatic index for the working fluid at the inlet segment - kinl; Pressure and temperature of the working fluid in the supra-piston cavity at the moment of opening the exhaust valves - pe, Te.

- Calculation of the values of parameters that do not change throughout the entire calculation cycle

Vh=3,144D2S,    Vc=Vh/(ε-1),

Fp=Fh=π4D2,

Cm=Sn/30, kexh1=1/kexh,   kexh2=(kexh-1)/kexh

βcr=2kexhi+2kexhikexhi-1, kinl1=1/kinl,   kinl2=(kinl-1)/kinl

dτi=dφi/(6n),

φexhLiftEnd=φ1+φexhLiftD±dφexhCorr

φexhEnd=φexhLiftEnd+φexhD±2dφexhCorr

n=φ4-φ1dφ+1

φinlLiftEnd=φ2+φinlLiftD±dφinlCorr

φinlD=φinlLiftEnd+φinlD.±2dφinlCorr

Vi=Ve

- Vp(i+1)=Vc+0,5Vh1-cosφi+dφi+S8L1-cos2(φi+dφi),

dVpi=Vp(i+1)-Vpi

- Checking the conditions for choosing the calculation way φi>φexhLiftEnd or φi≤φexhLiftEnd

4.1. If the condition φi≤φexhLiftEnd is true, then

μexhifexhi=0,5(μexhifexhi)max1-cosφi-φ1φexhClosD ±dφexhCorr180

4.2. If the condition φi>φexhLiftEnd, is true, then check the conditions φi>φexhEnd or φi≤φexhEnd

4.2.1. If the condition φi≤φexhEnd is true, then μexhifexhi=0,5(μexhfexh)max

4.2.2. If the condition φi>φexhEnd, is true, then μexhifexhi=0,5(μexhfexh)max1+cosφi-φexhEndφexhClosD ±dφexhCorr180

- Checking the conditions for choosing the calculation way pexhpi>βcr or pexhpi≤βcr

5.1. If the condition pexhpi≤βcr, is true, then dGcExhi=pi/(RexhTi)(pexh/pi)1kexhiμexhifexhi2Rexhkexhikexhi-1Tidτ

5.2. If the condition pexhpi>βcr is true, then

dGcExhi=pi/(RexhTi)(pexh/pi)1kexhiμexhifexhi2kexhikexhi-1RTTi1-(pexhpi )kexhi-1kexhidτ

- Fci=πD0,5S1-cosφi+S8L1-cos2(φi)

αTi=128D0,26,18Cm0,8(10-5pi)0,8Ti-0,53

dQhi=αтiFpTi-Tp+FhTi-Th+FciTi-Tcidτ

- dpi=kexhipiVi1ρi-dGcExhi-kexhi-1kexhidQexhipi-dVpi

φi+1=φi+dφi,   pi+1=pi+dpi

Gi+1=Gi-dGcExhi

GcExhi+1=GcExhi +dGcExhi

Ti+1=pi+1Vi+1/(Gi+1Rexhi);

Qexhi+1=Qexhi+dQexhi;

- Checking the conditions φi+1>540° or φi+1≤540°

8.1. If φi+1>540° is true then Lexh(i+1)=Lexhi+(pi+0,5dpi)dVi

- Checking the conditions φi+1>φ2 or φi+1≤φ2

9.1. If φi+1>φ2 is true then Lexh(i+1)=Lexh=const

- The calculation of the exhaust process is completed at the crank angle value corresponding to the moment the inlet valve starts opening φi+1≥φ2

BEHAVIOUR

B = ((checkCycleNum;ENGINE_WORKCYCLE;newCycle) + !checkCycleNum.Delta),

ENGINE_WORKCYCLE = ((checkCrankshaftRotationAngle;

getCrankshaftRotationSin;

(STROKE1 + STROKE2 + STROKE3 + STROKE4);

changeCrankshaftRotationAngle1;ENGINE_WORKCYCLE) + !checkCrankshaftRotationAngle),

STROKE1=((isStroke1;STROKE1_CALCULATION)),

STROKE2 = (isStroke2;STROKE2_CALCULATION),

STROKE3 = (isStroke3; STROKE3_CALCULATION),

STROKE4 = (isStroke4; STROKE4_CALCULATION),

/* Stroke 1*/

STROKE1_CALCULATION =

(

INITIAL_AIR_PARAMETERS_CALCULATION ;

getInletValveCrossSectionalAreaOpen ;

(getExhaustValveCrossSectionalAreaClose + notGetExhaustValveCrossSectionalAreaClose);

GET_HEAT_TRANSFERRED_AMOUNT;

getCylinderSupraPistonVolumeNew; getCylinderSupraPistonVolumeDelta;

(GET_SubMASS_PRESSURE_CASE1 + GET_SubMASS_PRESSURE_CASE2 + GET_SubMASS_PRESSURE_CASE3 + GET_SubMASS_PRESSURE_CASE4 );

getTemperatureInCylinder;

GET_SUBSTANCES_IN_CYLINDER_DATA;

print1

),

GET_SubMASS_PRESSURE_CASE1 =

(isIntakeCase1;

getSubstanceMassFromInlet(air);

(getSubstanceParametersInCylinder1 + getSubstanceParametersInCylinder2);

getSubstanceDensityInCylinder;

getSubstanceVelocityToExhaustValve(workingMixture);

getSubstanceMassToExhaust(workingMixture);

getPressureInCylinder1;

recalculateIntakeParameters1

),

GET_SubMASS_PRESSURE_CASE2 =

(isIntakeCase2;

getSubstanceMassFromInlet(air) ;

(getSubstanceParametersInCylinder3);

getSubstanceDensityInCylinder;

getSubstanceVelocityToExhaustValve(workingMixture);

getSubstanceMassToExhaust(workingMixture);

getPressureInCylinder2;

recalculateIntakeParameters2

),

GET_SubMASS_PRESSURE_CASE3 =

(isIntakeCase3;

getSubstanceMassFromInlet(air) ;

getPressureInCylinder3;

recalculateIntakeParameters3;

(getWorkStroke1T1 + getWorkStroke1T2)

),

GET_SubMASS_PRESSURE_CASE4 =

(isIntakeCase4;

(getSubstanceParametersInCylinder3);

getSubstanceDensityInCylinder;

getSubstanceVelocityToInletValve(workingMixture);

getSubstanceMassToInlet(workingMixture);

getPressureInCylinder4;

recalculateIntakeParameters4

),

INITIAL_AIR_PARAMETERS_CALCULATION =

(

getAirDensityInInlet .

getSubstanceVelocityFromInletValve(air)

),

GET_HEAT_TRANSFERRED_AMOUNT =

(getCylinderWallSurfaceArea;

getHeatTransferCoef;

getHeatTransferredAmount

),

/* Stroke 2*/

STROKE2_CALCULATION = (

(getInletValveCrossSectionalAreaClose + notGetInletValveCrossSectionalAreaClose);

getCylinderWallSurfaceArea; (getHeatTransferCoefS2 + getHeatTransferCoefS2CombStart); getHeatTransferredAmount;

getCylinderSupraPistonVolumeNew; getCylinderSupraPistonVolumeDelta;

(AIR_INTAKE_PROCESS_ENDING + COMPRESSION_PROCESS + (isCombustionStart;COMBUSTION_PROCESS));

print2),

AIR_INTAKE_PROCESS_ENDING = (

(notInletValveClose;

(GET_SubMASS_PRESSURE_CASE3 + GET_SubMASS_PRESSURE_CASE4);

getTemperatureInCylinder;

GET_SUBSTANCES_IN_CYLINDER_DATA

)

),

COMPRESSION_PROCESS =

(

(notInjectionStart;

getCombustionProductsVolumeFraction1;

getNewWMHeatCapacityRatio1;

getPressureInCylinderS2;

recalculateStroke2Parameters1;

getTemperatureInCylinder;

getCompressionWork1

) +

(isInjectionStart;

getFuelMassInCylinder;

getWorkMixtureAmountInCylinder2;

getCombustionProductsVolumeFraction1;

getNewWMHeatCapacityRatio1;

getPressureInCylinderS2;

recalculateStroke2Parameters1;

getTemperatureInCylinder;

getCompressionWork1)

),

COMBUSTION_PROCESS = (

getFuelBurnedFraction;

getFuelBurnedFractionD;

getReleasedHeatD;

getExcessAirCoeff;

getNeededAirAmount;

getChemicalMolecularChangeCoeff;

getMolecularChangeCoeff;

getCombProdAmountInCylinderNew;

getCombustionProductsVolumeFraction2;

(checktemperature1.getNewWMHeatCapacityRatio1 + checktemperature2.getNewWMHeatCapacityRatio2);

getPressureInCylinderS22;

recalculateStroke2Parameters2;

getTemperatureInCylinder2

),

GET_SUBSTANCES_IN_CYLINDER_DATA =

(

getResidualGasesCoefficient;

getWorkMixtureAmountInCylinder;

getAirAmountInCylinder;

getCombProdAmountInCylinder;

getPressureVolumeInletClose;

getCombustionProductsVolumeFraction2

),

/* Stroke 3*/

STROKE3_CALCULATION = (

getCylinderWallSurfaceArea; (getHeatTransferCoefS2CombStart); getHeatTransferredAmount;

getCylinderSupraPistonVolumeNew; getCylinderSupraPistonVolumeDelta;

((notCombustionEnd;COMBUSTION_PROCESS) + (isCombustionEnd;EXPANSION_PROCESS) + (isExhaustValveOpen;EXHAUST_PROCESS));

print3

),

EXPANSION_PROCESS =(

(checktemperature1.getNewWMHeatCapacityRatio1 + checktemperature2.getNewWMHeatCapacityRatio2);

getPressureInCylinderS22;

recalculateStroke2Parameters2;

getTemperatureInCylinder2

),

EXHAUST_PROCESS =

(getExhaustValveCrossSectionalAreaOpen;

((isCriticalPressureRatio;getSubstanceMassToExhaustS41(workingMixture)) +

(notCriticalPressureRatio;getSubstanceMassToExhaustS42(workingMixture)));

GET_HEAT_TRANSFERRED_AMOUNT;

getPressureInCylinderExhProcess;

recalculateStroke3Parameters;

getTemperatureInCylinderExhProcess

),

/* Stroke 4*/

STROKE4_CALCULATION = (

getCylinderSupraPistonVolumeNew; getCylinderSupraPistonVolumeDelta;

((notInletValveOpen;EXHAUST_PROCESS) +

(isInletValveOpen; getInletValveCrossSectionalAreaOpen;START_AIR_INTAKE_PROCESS));

print4

),

START_AIR_INTAKE_PROCESS =

(GET_HEAT_TRANSFERRED_AMOUNT;

INITIAL_AIR_PARAMETERS_CALCULATION;

getSubstanceMassFromInlet(air);

(getSubstanceParametersInCylinder1 + getSubstanceParametersInCylinder2);

getSubstanceDensityInCylinder;

getSubstanceVelocityToExhaustValve(workingMixture);

getSubstanceMassToExhaust(workingMixture);

getPressureInCylinder1;

recalculateIntakeParameters1;

getTemperatureInCylinder

)

initial data

crankshaftRotationAngle == -360 &&

initialCrankshaftRotationAngle == -360  &&

engineSpeed == 350*piConst/30 &&

dCrankshaftRotationAngle == 2*piConst/30 &&

crankshaftRotationFrequency == 350 &&

crankRadius == 0.29 &&

crankOffsetLength == 0 &&

connectingRodLength == 1.374 &&

crankCorrectionAngle == 0 &&

cylinderDiameter == 0.46 &&

compressionRatio == 14 &&

pistonStroke == 2*crankRadius &&

pistonClearance == 0.0008 &&

averagePistonSpeed == 4*crankRadius*crankshaftRotationFrequency &&

pistonTemperature == 673 &&

cylinderHeadTemperature == 723 &&

cylinderWallSurfaceTemperature == 523 &&

pistonArea == POW(cylinderDiameter,2)*piConst/4 &&

cylinderHeadArea == POW(cylinderDiameter,2)*(piConst/4) + piConst*cylinderDiameter*pistonClearance &&

totalCylinderVolume == POW(cylinderDiameter,2)*piConst*crankRadius/2 &&

combustionChamberVolume == (POW(cylinderDiameter,2)*piConst*crankRadius/2)/(compressionRatio-1)  &&

cylinderSupraPistonVolume == (POW(cylinderDiameter,2)*piConst*crankRadius/2)/(compressionRatio-1) && /* The initial value of the volume of the cylinder's supra-piston cavity is equal to the volume of the combustion chamber */

/*Valves parameters*/

intakeManifoldVolume == 3.210 &&

inletValveDiameter == 0.165 &&

inletValveOpenAngle == 311.8 &&

inletValveCloseAngle == -154.2  &&

inletValveOpenDuration == 56 &&

inletValveCloseDuration == 56 &&

maxInletValveCrossSectionalArea == POW(inletValveDiameter,2)*piConst/4 &&

exhaustManifoldVolume == 2.350 &&

exhaustValveDiameter == 0.16 &&

exhaustValveOpenAngle == 127.8 &&

exhaustValveCloseAngle == -314.5 &&

exhaustValveOpenDuration == 101 &&

exhaustValveCloseDuration == 101 &&

maxExhaustValveCrossSectionalArea == POW(exhaustValveDiameter,2)*piConst/4 &&

correctionAngle == 5 &&

fuelInjectionStartAngle == -5 &&

combustionDuration == 50 &&

air.temperatureInInletValve == 303.3538833039938 &&

air.gasConstant ==  287.2  &&

air.massInCylinder == 0 &&

air.heatCapacityRatio == 1.4 &&

air.massInInletValve == (pressureInInletValve*3*(POW(cylinderDiameter,2)*piConst*crankRadius/2))/(air.gasConstant*air.temperatureInInletValve) &&

air.density == pressureInInletValve/(air.gasConstant*air.temperatureInInletValve) &&

combustionProducts.gasConstant == 290 &&

combustionProducts.heatCapacityRatio == 1.25 &&

combustionProducts.temperatureInCylinder == 873.15 &&

combustionProductsMass == (pressureInCylinder*((POW(cylinderDiameter,2)*piConst*crankRadius/2)/(compressionRatio-1))) /(combustionProducts.gasConstant*temperatureInCylinder) &&

combustionProducts.massInCylinder == (pressureInCylinder*((POW(cylinderDiameter,2)*piConst*crankRadius/2)/(compressionRatio-1))) /(combustionProducts.gasConstant*temperatureInCylinder)  &&

workingMixture.temperatureInExhaustValve == 812.1211986734698 &&

workingMixture.gasConstant == 290 &&

workingMixture.density == pressureInCylinder/(workingMixture.gasConstant*temperatureInCylinder) &&

workingMixture.massInCylinder == (pressureInCylinder*((POW(cylinderDiameter,2)*piConst*crankRadius/2)/(compressionRatio-1))) /(combustionProducts.gasConstant*temperatureInCylinder) &&

fuelCyclicSupply == 25 &&

fuelCombustionLowestHeat == 42.7*POW(10,6) &&

fuel.density == 830 &&

fuel.massInCylinder == fuelCyclicSupply*fuel.density/POW(10,6) &&

WMvelocityTangentialComponent == 10 &&

pressureInCylinder == 180*POW(10,3) &&

temperatureInCylinder == 873.15 &&

pressureInInletValve == 220204.56112840094 &&

pressureInExhaustValve == 115683.93706653634 &&

stoichiometricAirFuelRatio == 14.5 &&

/* mass fraction of an element in the fuel */

H == 0.125 && O == 0.005 && C == 0.87 &&

oxygenInAirCoef == 0.21 &&

airNeededAmountForCombustion == (1/oxygenInAirCoef)*((C/12) + (H/4) + (O/32)) &&

time==1 && m==0 && n==0 && k==0 &&

strokeNum == 1 && rotationNum == 1 && engineWorkTime == 3 &&

coefA1 == 3.653 && coefA2 == 1.957 && coefB1 == -1.337*POW(10,-3) && coefB2 == 0.368*POW(10,2) &&

coefA12 == 2.415 && coefA22 == 1.287 && coefB12 == 1.257*POW(10,-3) && coefB22 == 	0.456*POW(10,2) &&

coefIgnitionA == 2*POW(10,-6) &&

coefIgnitionB == 1500 &&

coefIgnitionN == 1